4MP HD IR VF Dome Network Camera

IPC3534LB-ADZK-G

Key Features

- High quality image with 4MP, 1/3"CMOS sensor

- 4MP (2688*1520)@ 25/20fps; 4MP (2560*1440)@ 25/20fps; 3MP (2304*1296)@ 30/25fps;2MP (1920*1080)@30/25fps

- Ultra 265, H.265, H.264, MJPEG

- 120dB true WDR technology enables clear image in strong light scene

- Support 9:16 Corridor Mode

- Built-in Mic

- Smart IR, up to 40m (131ft) IR distance

- Supports 256 G Micro SD card

- IK10 vandal resistant

- IP67 protection

- Support PoE power supply

- 3-Axis

Specifications

|  | IPC3534LB-ADZK-G |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Camera |  |  |  |  |  |  |  |  |
| Sensor | 1/3", 4.0 megapixel, progressive scan, CMOS |  |  |  |  |  |  |  |
| Lens | 2.8 ~ 12mm, AF automatic focusing and motorized zoom lens |  |  |  |  |  |  |  |
| DORI Distance | Lens (mm) |  | Detect (m) |  | Observe (m) | Recognize (m) |  | Identify (m) |
|  | 2.8 |  | 42 |  | 16.8 | 8.4 |  | 4.2 |
|  | 12 |  | 180 |  | 72 | 36 |  | 18 |
| Angle of View (H) | 102.79°~ 30.86° |  |  |  |  |  |  |  |
| Angle of View (V) | 54.50°~ 17.49° |  |  |  |  |  |  |  |
| Angle of View (O) | 106.94°~35.49° |  |  |  |  |  |  |  |
| Adjustment angle | Pan: 3° ~ 360° |  |  | Tilt: 0° ~ 70° |  |  | Rotate: 3° ~ 360° |  |
| Shutter | Auto/Manual, 1 ~ 1/100000s |  |  |  |  |  |  |  |
| Minimum Illumination | Colour: 0.003Lux (F1.6, AGC ON) 0Lux with IR |  |  |  |  |  |  |  |
| Day/Night | IR-cut filter with auto switch (ICR) |  |  |  |  |  |  |  |
| Digital Noise Reduction | 2D/3D DNR |  |  |  |  |  |  |  |
| S/N | >56dB |  |  |  |  |  |  |  |
| IR Range | Up to 40m (131ft) IR range |  |  |  |  |  |  |  |
| Wavelength | 850nm |  |  |  |  |  |  |  |
| IR On/Off Control | Auto/Manual |  |  |  |  |  |  |  |
| Defog | Digital Defog |  |  |  |  |  |  |  |
| WDR | 120dB |  |  |  |  |  |  |  |
| Video |  |  |  |  |  |  |  |  |
| Video Compression | Ultra 265, H.265, H.264, MJPEG |  |  |  |  |  |  |  |
| H.264 code profile | Baseline profile, Main profile, High profile |  |  |  |  |  |  |  |
| Frame Rate | Main Stream: 4MP (2688*1520), Max 25fps; 4MP (2560*1440), Max 25fps; 3MP (2304*1296), Max 30fps; 1080P (1920*1080), Max 30fps; Sub Stream: 720P (1280*720), Max 30fps; D1 (720*576), Max 30fps; 640*360,Max 30fps; 2CIF(704*288), Max 30fps; CIF(352*288), Max 30fps; |  |  |  |  |  |  |  |
| Video Bit Rate | 128 Kbps~16 Mbps |  |  |  |  |  |  |  |
| 9:16 Corridor Mode | Supported |  |  |  |  |  |  |  |
| OSD | Up to 4 OSDs |  |  |  |  |  |  |  |
| Privacy Mask | Up to 4 areas |  |  |  |  |  |  |  |
| ROI | Up to 8 areas |  |  |  |  |  |  |  |
| Motion Detection | Up to 4 areas |  |  |  |  |  |  |  |
| Image |  |  |  |  |  |  |  |  |
| White Balance |  | Auto/Outdoor/Fine Tune/Sodium Lamp/Locked/Auto2 |  |  |  |  |  |  |
| Digital Noise Reduction |  | 2D/3D DNR |  |  |  |  |  |  |
| Smart IR |  | Supported |  |  |  |  |  |  |
| Flip |  | Normal/Vertical/Horizontal/180°/90°Clockwise/90°Anti-clockwise |  |  |  |  |  |  |
| Dewarping |  | N/A |  |  |  |  |  |  |
| HLC |  | Supported |  |  |  |  |  |  |
| BLC |  | Supported |  |  |  |  |  |  |
| Audio |  |  |  |  |  |  |  |  |
| Audio Compression | G.711 |  |  |  |  |  |  |  |
| Suppression | Supported |  |  |  |  |  |  |  |
| Sampling Rate | 8KHZ |  |  |  |  |  |  |  |
| Storage |  |  |  |  |  |  |  |  |
| Edge Storage | Micro SD, up to 256GB |  |  |  |  |  |  |  |
| Network Storage | ANR, NAS(NFS) |  |  |  |  |  |  |  |
| Smart |  |  |  |  |  |  |  |  |
| Behavior Detection | Cross line, intrusion(based on human body detection) |  |  |  |  |  |  |  |
|  | motion detection, tampering alarm, audio detection |  |  |  |  |  |  |  |
| General Function | Watermark, IP address filtering, access policy, ARP protection, RTSP authentication, user authentication |  |  |  |  |  |  |  |
| Network |  |  |  |  |  |  |  |  |
| Protocols | IPv4, IGMP, ICMP, ARP, TCP, UDP, DHCP, RTP, RTSP, RTCP, DNS, DDNS, NTP, FTP, UPnP, HTTP, HTTPS, SMTP, SSL, QoS, 802.1x, SNMP |  |  |  |  |  |  |  |
| Compatible Integration | ONVIF (Profile S, Profile G, Profile T), API |  |  |  |  |  |  |  |
| Client | EZStation EZView EZLive |  |  |  |  |  |  |  |
| Web Browser | Plug-in required live view: IE9+, Chrome 41 and below, Firefox 52 and below |  |  |  |  |  |  |  |
|  | Plug-in free live view: Chorme 57.0+, Firefox 58.0+, Edge 16+, Safari 11+ |  |  |  |  |  |  |  |
| Interface |  |  |  |  |  |  |  |  |
| Network | 10M/100M Base-TX Ethernet |  |  |  |  |  |  |  |
| Built-in Mic | Supported |  |  |  |  |  |  |  |
| Certifications |  |  |  |  |  |  |  |  |
| Certifications | CE: EN 60950-1 UL: UL60950-1 FCC: FCC Part 15 |  |  |  |  |  |  |  |
| General |  |  |  |  |  |  |  |  |
| Power | DC 12V±25%, PoE (IEEE 802.3af) |  |  |  |  |  |  |  |
|  | Power consumption: Max 7.5W |  |  |  |  |  |  |  |
| Dimensions (Ø x H) | Φ128mmx100mm (Ø5.0” x 3.9”) |  |  |  |  |  |  |  |
| Net Weight | 0.65kg (1.43lb) |  |  |  |  |  |  |  |
| Material | Metal + Plastic |  |  |  |  |  |  |  |
| Working Environment | -30°C ~ 60°C (-22°F ~ 140°F), Humidity: ≤95% RH (non-condensing) |  |  |  |  |  |  |  |
| Strorage Environment | -30°C ~ 60°C (-22°F ~ 140°F), Humidity: ≤95% RH (non-condensing) |  |  |  |  |  |  |  |
| Surge Protection | 6KV |  |  |  |  |  |  |  |
| Ingress Protection | IP67 |  |  |  |  |  |  |  |
| Vandal Resistant | IK10 |  |  |  |  |  |  |  |
| Reset Button | Supported |  |  |  |  |  |  |  |

Dimensions

Accessories

TR-JB03-H-IN       TR-JB07/WM03-F-IN        TR-WM03-B-IN         TR-UP06-IN       TR-A01-IN

TR-SE24-IN                     TR-SE24-A-IN                     TR-CM24-IN

| Junction Mount | Pole Mount |
| --- | --- |
| TR-JB03-H-IN | TR-JB07/WM03-F-IN(TR-WM03-B-IN)+TR-UP06-IN |
| TR-CM24-IN+TR-JB03-H-IN |  |

Zhejiang Uniview Technologies Co., Ltd.

Building No.10, Wanlun Science Park, Jiangling Road 88, Binjiang District, Hangzhou, Zhejiang, China (310051)

Email: overseasbusiness@uniview.com; globalsupport@uniview.com

http://www.uniview.com

©2021 Zhejiang Uniview Technologies Co., Ltd. All rights reserved.

*Product specifications and availability are subject to change without notice.